SMART WASHROOM

Hygiene Monitoring & Automatic Disinfection System — One-Page Overview

## What it is

A single retrofit device — built around an ESP32 — that installs inside an existing public washroom with no structural changes. It monitors air quality and occupancy, automatically disinfects the seat after every use, and keeps a live Streamlit dashboard of tank levels and status.

## The problem it solves

Public washrooms suffer from odor, delayed cleaning, and unknown consumable levels, but existing “smart toilet” solutions require full reconstruction. This is a low-cost, plug-and-play alternative.

## The technical core (what makes it worth a paper / patent)

Virtual sensing: the disinfectant tank has no physical level sensor. Its remaining level is inferred by a trained model from indirect signals (spray counts, timing, usage history) instead — evaluated against a naive counter baseline — and that estimate drives a real alert/actuator, not just a predicted number.

## How it works, in one line per stage

| Sense | LD2410 + IR (occupancy, cross-checked), MQ135 (air quality), HC-SR04 (water level) |
| --- | --- |
| Decide | Occupancy-gated rule: never spray while occupied; spray on exit; capped extra sprays if air stays bad; flag for manual checkup if spraying doesn't help |
| Act | Mist pump disinfects the seat; a second pump auto-refills the water tank from a backup reservoir |
| Learn | Anomaly detection on air quality, virtual sensing for disinfectant level, usage clustering across cubicles |

## Status

Hardware partially built. ML pipeline validated on a 4-cubicle synthetic dataset (100k+ rows) with realistic depletion, noise, and a verified safety guarantee (zero disinfection events while occupied). Streamlit is the primary interface for now; a physical OLED display is Phase 2.

## Full detail

See the companion Project Report (hardware/system design) and Features & Workflow document (ML/software architecture) for complete detail, open items, and the dataset/generator files.