# Features & Workflow

IoT + ML Smart Washroom System — architecture and workflow blueprint for the software side

Revision 2 — companion to the Project Report; hand this to whoever builds the Streamlit app

# 1. Complete System Tech Stack

## Hardware Layer

- ESP32 microcontroller

- MQ-135 gas sensor (odor / air quality)

- LD2410 mmWave presence sensor + IR/PIR motion sensor — used together, not either/or

- HC-SR04 ultrasonic sensor (primary water tank level)

- 4-channel relay: CH1 mist pump, CH2 water-refill pump

- Mist maker + spray pump, water-refill pump (previously idle, now assigned a job), backup water reservoir

## Communication / Broker

ESP32 sends JSON payloads over Wi-Fi (MQTT or HTTP REST) to a backend (Firebase / Supabase / a simple Flask API), which Streamlit reads.

## Software & Web Dashboard

Python, Streamlit (UI), Pandas / NumPy (data processing), Plotly / Altair (charts).

## Machine Learning Libraries

scikit-learn (Isolation Forest for anomaly detection; Random Forest / Gradient Boosting for the virtual-sensing regressor), optionally a small LSTM/GRU for the DL angle, K-Means for usage clustering.

# 2. The ML / Logic Features — what's really ML and what isn't

Note: Being upfront about this matters for both the paper and the demo: three of these are genuine ML; one is a safety-critical rule-based controller and should be labelled as such, not counted as a fourth model.

## Feature 1: Anomaly Detection (Air Quality)

What it does: learns what “normal” MQ135 behaviour looks like across hours/usage rather than a fixed ppm cutoff, and flags unusual spikes (chemical spill, blocked drain, or a failing sensor).

Model: Isolation Forest or One-Class SVM (scikit-learn), trained on engineered features — rate of change, rolling mean/std, time-of-day — not the raw ppm value alone.

A good use for this model's output: flag the case in Section 3 below where repeated extra sprays don't bring the gas reading down — that pattern is a strong anomaly signal (probable sensor/ventilation fault) rather than a genuinely dirty seat.

## Feature 2: Virtual Sensing + Predictive Refill Forecasting (core contribution)

What it does: the disinfectant tank has no dedicated level sensor (see Project Report, Section 6) — its remaining level is inferred from indirect signals: spray-event counts, timing, and usage history. The water tank does have a real sensor (HC-SR04), so its refill is a straightforward automatic trigger; the disinfectant estimate is the genuinely novel piece.

Model: Random Forest / Gradient Boosting regressor as the primary baseline, benchmarked against a naive “spray count × average volume per spray” counter — the gap between the two is the actual result worth reporting. A small LSTM/GRU on the windowed sequence is a reasonable stretch goal if there's time.

Forecasting “hours until refill needed” is a straightforward extrapolation of this same estimate. Evaluate with a time-based train/test split (e.g., train on the first ~60 days, test on the rest) — a random row split will leak badly since adjacent 5-minute readings are highly correlated.

## Feature 3: Disinfection Control (rule-based, occupancy-gated — not ML)

Note: This was revised after design review: the original trigger (gas ppm over a threshold) did not check occupancy, so it could fire while someone was still inside. The corrected logic below is safety-critical and should stay a deterministic rule, not a learned model, so its behaviour is always predictable and auditable.

- If either occupancy sensor reads “occupied” → never trigger the mist pump.

- Occupied → Vacant transition → baseline spray (5 seconds).

- Still poor air quality a short while after that, and still vacant → up to 2 extra sprays (8 seconds each), each re-checking vacancy first.

- Still poor after 2 extra sprays → CONFIRMED: stop spraying and set needs_manual_checkup = 1 (a dashboard alert for staff), instead of continuing to spray.

- No exit event for 4+ hours and currently vacant → one idle-refresh spray.

Note: In the simulation, needs_manual_checkup fires fairly often for the busier cubicle profiles under the current 2-spray cap — worth watching once real data exists, and easy to loosen (raise the cap) if it turns out to be over-triggering rather than catching real faults.

## Feature 4: Fleet & Usage Analytics

What it does: aggregates data over days/weeks to find peak usage times and traffic patterns.

Model: K-Means clustering on hourly traffic, grouped by cubicle.

Note: “Fleet” requires more than one unit's worth of data to mean anything. The synthetic dataset now includes 4 cubicles with genuinely different traffic shapes (not just scaled volume) for this reason — but the physical prototype is currently one unit, so this feature is trained on synthetic multi-unit data until more real hardware exists.

# 3. Streamlit Website Architecture

Three tabs, as before, updated for the corrected sensor and actuator set:

## Tab 1: Live Monitoring

- Metrics row: People Count Today, Current Air Quality (ppm), Primary Water Tank (%), Disinfectant Level — virtual sensing estimate (%), Mist Maker Status

- Occupancy row: shows both LD2410 and IR readings side by side, plus the combined “either sensor says occupied” status actually used for control — useful for spotting sensor disagreement

- Water Tank Visual: progress bar from HC-SR04; badge showing when the refill pump (CH2) is auto-running from the backup reservoir

- Disinfectant Visual: progress bar from the virtual-sensing estimate; badge for “Low — manual refill needed” (this one is a staff alert, not an automatic action — see Project Report Section 9)

- Live Activity Feed: e.g. “10:14 AM — Occupied → Vacant, baseline spray fired” / “10:22 AM — extra spray (1 of 2)” / “14:03 — water refill pump ON”

## Tab 2: Predictive Maintenance & ML Insights

- Disinfectant Forecast Card: “At current usage, disinfectant will need a manual refill in ~X hours”, with the model estimate shown alongside the naive-counter baseline for comparison

- Anomaly Alert Section: flags from the Isolation Forest model, plus needs_manual_checkup events raised directly by Feature 3's spray-cap logic

- Sensor Agreement Panel: how often LD2410 and IR disagree — a rising disagreement rate is itself worth watching (possible sensor drift)

## Tab 3: Historical Analytics & Traffic Patterns

- Peak Traffic Heatmap: footfall by hour, per cubicle

- Air Quality Trends: MQ135 before vs. after each spray event

- Resource Consumption: sprays per day, water-refill events per day, estimated disinfectant consumed per day

# 4. End-to-End Data & Hardware Workflow

## Hardware Layer

LD2410 and IR both read continuously; MQ135 reads continuously; HC-SR04 reads the primary tank.

## ESP32 Brain

- Occupied (either sensor) → no actuation, regardless of any other reading.

- Occupied → Vacant transition → Relay CH1 (mist pump) ON for 5s, then OFF.

- Air still poor after that and still vacant → up to 2 more capped sprays; otherwise stop and flag.

- Water level below threshold → Relay CH2 (refill pump) ON until the primary tank is topped up from the backup reservoir.

- Packs all values into JSON every 3–5 seconds:

{"occupancy_ld2410": 0, "motion_ir": 0, "gas_ppm": 210, "water_level_cm": 14.2, "disinfectant_pct_est": 58.4, "mist_status": 0, "water_refill_status": 0, "disinfectant_alert": 0}

## Communication Layer

ESP32 → Wi-Fi → Firebase / MQTT / simple REST endpoint, every 3–5 seconds.

## Streamlit Backend & ML Processing

- Reads incoming JSON.

- Runs Isolation Forest → flags gas anomalies.

- Runs the virtual-sensing regressor → updates the disinfectant-level estimate and the refill countdown.

- Checks the rule-based Feature 3 log → surfaces any “repeated spray, no improvement” fault.

## Streamlit Frontend UI

Updates live charts, tank levels, and alert banners in real time.

# 5. Summary Checklist for Software Teammate

- Backend: Firebase / Supabase / a simple Flask or FastAPI endpoint for the ESP32 to POST sensor JSON to.

- Streamlit layout: st.columns() for the metric cards, st.tabs() for the three sections above.

- ml_models.py: Isolation Forest (anomaly), the virtual-sensing regressor + naive-counter baseline (disinfectant), K-Means (usage clustering).

- Auto-refresh: streamlit-autorefresh so the dashboard updates without manual reloads.

- Display both occupancy sensors AND the combined status used for control — don't collapse them into one value in the UI, since sensor disagreement is itself useful information.

- Clearly label the disinfectant refill badge as a staff alert, not an automatic action, so nobody assumes the system refills itself.

# 6. Data Dictionary — washroom_dataset_multi_cubicle.csv

All 17 columns, for anyone reading the dataset without the full chat history behind it.

| Column | Meaning |
| --- | --- |
| timestamp | 5-minute-resolution reading |
| cubicle_id | Which simulated cubicle (4 distinct traffic profiles) |
| mq135_gas_ppm | Air-quality / odor sensor reading |
| occupancy_ld2410 | mmWave presence sensor (catches a stationary person) |
| motion_ir | PIR motion sensor (misses a stationary person ~30% of the time) |
| water_level_cm | Primary water tank level (real HC-SR04 sensor) |
| disinfectant_level_virtual_pct | Disinfectant level — virtually sensed, no physical sensor exists |
| mist_maker_status | 1 = disinfectant spray firing this row |
| needs_manual_checkup | 1 = repeated spraying failed to clear the air; flagged for a person, stays 1 until air quality actually recovers |
| water_refill_status | 1 = water refill pump firing this row (automatic) |
| disinfectant_refill_status | 1 = disinfectant crossed the low threshold (manual staff top-up, not automatic) |
| refill_motor_status | 1 if either refill above fired — kept for backward compatibility |
| entry_count | Visits so far today (resets at midnight) |
| hours_since_seat_spray | Time since the last mist spray |
| hours_since_deep_clean | Time since the daily 5 AM janitorial deep clean |
| hygiene_score | Synthetic 0–100 proxy label, not a real measurement (see Open Items) |
| needs_cleaning | Synthetic classification label derived probabilistically from hygiene_score |